Created by Grzegorz Raczka, last modified by Piotr Hudaszek on Mar 05, 2026

Contents

Motor controlling commands	1

=MJJ <float1> <float2>	1

=?MxC	2

=MxA <float>	2

=MxR <float>	2

=MxJ <float>	2

=MJS	2

=?MJS	2

=MxS	2

=?MxS	2

=?MxJ	3

=MxV <float>	3

=?MxV	3

=MxL <float1> <float2>	3

=?MxL	3

=MxP <float1> <float1> <float3> <float4> <float5> <float6> <float7> <float8>	3

=?MxP	3

=MxU <integer>	3

=?MxU	3

=MxD <float1> <optional_float2>	3

=?MxD	3

=MxF <integer>	3

=MxX <integer>	4

=?MxX	4

=MxW <float>	4

=?MxW	4

=MxC <float>	4

=?MxE	4

=?MxT	4

=?MxI	4

=?MxA	4

=?MxR	4

Homing	4

=MJV <float1> <float2> <optional_float3>	4

=?MJV	4

=MJH <optional_integer>	4

=?MJH	4

=MxH <optional_integer>	5

=?MxH	5

=MxO <float>	5

=?MxO	5

=MxK <float>	5

=?MxK	5

=HDE <integer>	5

=?HDE	5

Cold start	5

=SLZ <integer>	5

=?SLZ	5

=SLC <float>	5

=?SLC	6

=SLW <float>	6

=?SLW	6

System commands	6

=SLM <integer>	6

=?SLM	6

=SLS <integer>	6

=?SLS	6

=SL!	6

=SLD	6

=SLR	6

=?SLR	6

=SLB	6

=?SLB	6

=?SLV	6

=SLF <integer>	6

=?SLF	7

=?SLT	7

=SLT <float>	7

=?SLI	7

=MJR	7

=MJD	7

=?SLH	7

=SLN <ip> <gateway> <mask> <port>	7

=?SLN	7

=?SEV	7

Power channel control commands	7

=PxC <integer>	7

=?PxC	7

=P2V <float>	7

=?PxV	7

=?PxI	7

=?PxT	8

Half bridge outputs	8

=BcC <char>	8

=?BcC	8

=?BcI	8

=?BcV	8

=?BcT	8

=BcD <integer>	8

=?BcD	8

Optically isolated channels	8

=OcC <integer>	8

=?OcC	9

=OcD <integer>	9

=?OcD	9

Fan control commands	9

=?F1V	9

=?F1I	9

=F1C <integer>	9

=?F1C	9

=F1D <integer>	9

=?F1D	9

XIO outputs control commands	9

=XcS <integer>	9

=?XcS	9

Debugging commands	10

=MxM <integer1> <integer2>	10

=?MxM <integer>	10

=PSE	10

=RSE	10

=PMD	10

=SFT <float>	10

Each command must start with '=' sign. Current serial port baudrate is 800000bps. Each command's parameter is separated by space.

You should get a response for each command. For example "=?MPV" command

[2181312] 1628 D: ? M P V 100.000 \C8

If you do a mistake or provide unsupported command you will see for example:

[2280371] 1630 N: M K V 10 \79

which means that this request was ignored (message type = N).

The format is

[<timestamp>] <UPID> <message type>: <command>

timestamp is a millisecond clock

UPID is a unique packet id. This is an integer number increased by 1 for every command.

Message type is a single character and it could be:

- none → "space"

- info → I

- warning → W

- critical error → E

- checksum error → C

- storage error → S

- wait → T (extra time is needed for execution)

- renewed → R

- done → D

- ignored → N

- motor not initialized → M

characters printed at the end of line eg. "\C8" are a checksum

# Motor controlling commands

"x" in command name means "P" or "T" depending on axis. "P" is for pan and "T" is for tilt

| ### =MJJ <float1> <float2> | Jog movement for Pan and Tilt: float1 - is a velocity of the Pan axis movement in float number format eg. 60.0 (unit: degrees per second) float2 - is a velocity of the Tilt axis movement in float number format eg. 60.0 (unit: degrees per second) The sign of the velocity determines direction. eg. "=MJJ 60.0 60.0" - velocity > 0.0 → Pan rotates right or Tilt goes up. - velocity < 0.0 → Pan rotates left or Tilt goes down. Requested motion will last 4 seconds only, so for longer movement this command must be repeated. Motorcontrol already does this by repeating command for every 2 seconds. |
| --- | --- |
| ### =?MxC | Prints current angle: (MPC for pan or MTC for tilt) |
| ### =MxA <float> | Move to the specific angle provided as <float> parameter for pan (MPA) or tilt (MTA) eg. "=MPA 120.0" |
| ### =MxR <float> | Change current angle by a difference provided as <float> parameter for pan (MPR) or tilt (MTR) eg. "=MPR -12.4" Example: if the current tilt position is 11.0 degrees then command "=MTR -4.0" will change this position to 7.0 degree |
| ### =MxJ <float> | Jog movement of the specific axis. "float" parameter is a velocity of the movement eg. 60.0 (unit: degrees per second) The sign of the velocity determines direction. - velocity > 0.0 → Pan rotates right (for MPJ) or Tilt goes up (for MTJ) - velocity < 0.0 → Pan rotates left (for MPJ) or Tilt goes down (for MTJ) Requested motion will last 4 seconds only, so for longer movement this command must be repeated. Motorcontrol already does this by repeating command for every 2 seconds. |
| ### =MJS | Stop both motors immediately. |
| ### =?MJS | Check if both motors are still. |
| ### =MxS | Stop motor for pan (MPS) or tilt (MTS) |
| ### =?MxS ### =?MxJ | Check if motor is still for specific axis. |
| ### =MxV <float> | Set maximum velocity provided by <float> parameter in degrees per second for specific axis: pan (MPV) or tilt (MTV) eg. "=MPV 120.0" |
| ### =?MxV | Get velocity target for pan (MPV) or tilt (MTV) |
| ### =MxL <float1> <float2> | Set axis limits for pan (MPL) or tilt (MTL). <float1> is an angle for lower limit and <float2> is an angle for upper limit. |
| ### =?MxL | Get axis limits. |
| ### =MxP <float1> <float1> <float3> <float4> <float5> <float6> <float7> <float8> | Set acceleration/deceleration profile for specific axis: pan (MPP) or tilt (MTP) - float1 → starting velocity [deg/s] - float2 → initial acceleration [deg/s^2] - float3 → initial target velocity [deg/s] - float4 → maximum acceleration [deg/s^2] - float5 → maximum velocity [deg/s] - float6 → maximum deceleration [deg/s^2] - float7 → deceleration for stopping [deg/s^2] - float8 → velocity for release [deg/s] |
| ### =?MxP | Get acceleration/deceleration profile for specific axis: pan (MPP) or tilt (MTP) |
| ### =MxU <integer> | Set update period for streaming positions of the specific axis. <integer> parameter is a period in milliseconds. |
| ### =?MxU | Get update period for streaming positions. |
| ### =MxD <float1> <optional_float2> | Set correction detection sensitivity for specific axis (MPD for pan or MTD for tilt). - float1 → mismatch sensitivity angle, smaller → higher sensitivity - optional_float2 → stall sensitivity angle, smaller → higher sensitivity (this is optional parameter and can be omitted) |
| ### =?MxD | Get correction detection sensitivity. |
| ### =MxF <integer> | Set flags. <integer> is a decimal number representing set and cleared bits of each flag. - bits 0-2 → homing mode - bit 3 → inverted motor direction - bit 4 → inverted encoder A/B phases - bit 5 → automatically adjust encoder direction during homing - bit 6 → inverted homing sensor output |
| ### =MxX <integer> | Set encoder ticks per revolution. |
| ### =?MxX | Get encoder ticks per revolution. |
| ### =MxW <float> | Set encoder constant. |
| ### =?MxW | Get encoder constant. |
| ### =MxC <float> | Set current angle to the provided value in <float> parameter. Motor will not move to such position but current position will be changed to provided angle. |
| ### =?MxE | Get current encoder angle. |
| ### =?MxT | Get motor controller temperatures. Will return two values: current temperature and peak temperature that will be cleared by the "?MxT" command |
| ### =?MxI | Get motor currents. Will return two values: actual current and peak current that will be cleared by the "?MxI" command |
| ### =?MxA ### =?MxR | Get target angle. |

# Homing

| ### =MJV <float1> <float2> <optional_float3> | Set homing and post-homing velocities. <float1> is a homing velocity and <float2> is a post-homing velocity. <float3> is a self-correction velocity which is an optional parameter and can be omitted. |
| --- | --- |
| ### =?MJV | Get homing and post-homing velocities. |
| ### =MJH <optional_integer> | Homing on both axes. Optional parameter can be a number 0 or 1. If it is set to 1 then debug messages will be printed during homing. |
| ### =?MJH | Get homing status 0 - homing failed 1 - homing finished successfully |
| ### =MxH <optional_integer> | x - is P or T (pan or tilt) Homing on specific axis. Optional parameter is an integer in range 1-6 which is a homing mode. Available homing modes: - 1 → manual - 2 → line - 3 → limits - 4 → strip - 5 → notch - 6 → Zline Default homing mode is a mode currently using by companion board. |
| ### =?MxH | Get homing mode for specified axis |
| ### =MxO <float> | Set offset angle after homing for specific axis. <float> is an angle offset in degrees |
| ### =?MxO | Get offset angle for specific axis. |
| ### =MxK <float> | Set temperature compensation offset for line homing for specific axis. <float> is a temperature compensation offset in degrees Celsius. |
| ### =?MxK | Get temperature compensation offset for specific axis. |
| ### =HDE <integer> | Set homing delay in seconds. |
| ### =?HDE | Get homing delay. Returned value is in seconds. |

# Cold start

| ### =SLZ <integer> | Set heater configuration. <integer> is a decimal number representing bits configuration of the heaters - bits 1:0 → 00: line not used, any other value: line is used - bits 3:2 → 00: line not used, any other value: line is used - bits 5:4 → 00: line not used, any other value: line is used - bits 7:6 → 00: line not used, any other value: line is used - bit 8 → O1: currently not used - bit 9 → O2: currently not used |
| --- | --- |
| ### =?SLZ | Get heater configuration. |
| ### =SLC <float> | Set cold start threshold temperature. |
| ### =?SLC | Get cold start threshold temperature. |
| ### =SLW <float> | Set keep desired warm temperature. |
| ### =?SLW | Get keep warm temperature. |

# System commands

| ### =SLM <integer> | Set system model. - 12 → AERON - 41 → JAEGAR V1 - 42 → JAEGAR V2 (this is the only one supported model) |
| --- | --- |
| ### =?SLM | Get system model. |
| ### =SLS <integer> | Set serial number. |
| ### =?SLS | Get serial number. |
| ### =SL! | Save current settings to the EEPROM. |
| ### =SLD | Restore default settings and save them to the EEPROM. |
| ### =SLR | System reset. |
| ### =?SLR | Get system up time. |
| ### =SLB | Reset communication buffers. |
| ### =?SLB | Get number of buffered commands. |
| ### =?SLV | Print firmware version. |
| ### =SLF <integer> | Set control flags. <integer> parameter is a decimal number represents bits of the specific feature: - bit 0 → cold start; 0: disabled, 1: enabled - bit 1 → keep warm; 0: disabled, 1: enabled - bits 2:7 → reserved - bit 8 → initial homing after reset - bit 9 → unsolicited self-correction messages - bit 10 → self-correction - bit 11 → position change streaming - bits 12:14 → number of payloads - bit 15 → homing delay; 0: disabled, 1: enabled |
| ### =?SLF | Get control flags. |
| ### =?SLT | Get local temperature. |
| ### =SLT <float> | Set local temperature. |
| ### =?SLI | Direct read from IMU. |
| ### =MJR | Set both motor controllers registers to default values. |
| ### =MJD | Set both motor controllers registers to default values and restore all axes settings to defaults. |
| ### =?SLH | Get humidity. |
| ### =SLN <ip> <gateway> <mask> <port> | Set network configuration. |
| ### =?SLN | Get network configuration. |
| ### =?SEV | Get system events. Command returns 3 numbers: - Cold start entered flag. - 1 if camera entered cold start during power up - 0 if camera wasn't in cold start - Homing delay finished flag. - 1 → homing delay timed out. Camera is not waiting before homing - 0 → homing delay is still in progress. Camera is waiting before start homing - Remaining time before homing. Returned value is in seconds. |

# Power channel control commands

"x" in command name is a number of channel (0, 1, 2 or 3). Some commands support only specific channels.

| ### =PxC <integer> | Control of power channel. (P1C for channel 1 or P2C for channel 2). |
| --- | --- |
| ### =?PxC | Get state of power channel (P1C for channel 1 or P2C for channel 2). |
| ### =P2V <float> | Set power output voltage to the value provided by <float> parameter. Command for channel 1 will be ignored. |
| ### =?PxV | Get power output voltage for specific channel. |
| ### =?PxI | Get power output current for specific channel. |
| ### =?PxT | Get power channel temperature. |

# Half bridge outputs

c → output channel. Supported channels are: 1, 2, 3, 4

| ### =BcC <char> | Set half bridge output for specific channel. <char> is a single character and can be set to "0" or "1". - "0" → output disabled - "1" → output enabled |
| --- | --- |
| ### =?BcC | Get half bridge output for specific channel. It may return: - "0" → output disabled - "1" → output enabled - "Z" → high impedance |
| ### =?BcI | Get half bridge current for specific channel. |
| ### =?BcV | Get half bridge voltage for specific channel. Each channel can return the same voltage (Vo1) if it's enabled. |
| ### =?BcT | Get half bridge temperature. Each channel returns the same temperature (it is the same chip). |
| ### =BcD <integer> | Set duty cycle for specific channel. <integer> is a duty cycle and supports values from 0 to 100. |
| ### =?BcD | Get duty cycle of the specific channel. |

# Optically isolated channels

c → output channel. Supported channels are: 1, 2

channel 1 controls pin "RPB8" (RLY1)

channel 2 controls pin "RPB9" (RLY2)

| ### =OcC <integer> | Set optically isolated output for specific channel. <integer> can be set to 0 or 1. - 0 - output disabled - 1 - output enabled |
| --- | --- |
| ### =?OcC | Get optically isolated output for specific channel. |
| ### =OcD <integer> | Set duty cycle for specific channel. <integer> is a duty cycle and supports values from 0 to 100. |
| ### =?OcD | Get duty cycle of the specific channel. |

# Fan control commands

| ### =?F1V | Get fan output voltage. There is only 1 channel of the fan output. Other channels will be ignored. |
| --- | --- |
| ### =?F1I | Get fan output current. There is only 1 channel of the fan output. Other channels will be ignored. |
| ### =F1C <integer> | Enable or disable fan based on <integer> parameter. "1" enables fan and "0" disables. There is only 1 channel of the fan output. Other channels will be ignored. |
| ### =?F1C | Get state of the fan channel. There is only 1 channel of the fan output. Other channels will be ignored. |
| ### =F1D <integer> | Set duty cycle of the fan channel based on <integer> parameter. Supported range is 0-100. There is only 1 channel of the fan output. Other channels will be ignored. |
| ### =?F1D | Get duty cycle f the fan channel. There is only 1 channel of the fan output. Other channels will be ignored. |

# XIO outputs control commands

| ### =XcS <integer> | Set value to an external pin. <integer> is a GPIO output state and can be set to 0 or 1. "c" is a pin number (0-7) that can be set to one of the following values: - 0 → XIO0 (LATG, 12) - 1 → XIO1 (LATG, 13) - 2 → XIO2 (LATE, 2) - 3 → XIO3 (LATE, 3) - 4 → XIO4 (LATG, 14) - 5 → XIO5 (LATE, 1) - 6 → XIO6 (LATE, 0) - 7 → XIO7 (LATA, 7) Example: "=X2S 1" sets XIO2 (LATE, 2) to high state. |
| --- | --- |
| ### =?XcS | Get input value of the specified pin. "c" can be set to 0-7. Refer to "XcS" command for details. |

# Debugging commands

| ### =MxM <integer1> <integer2> | Write to the TMC5160 for specific axis motor controller (MPM for pan or MTM for tilt). <integer1> is a register address and <integer2> is a value to be written. |
| --- | --- |
| ### =?MxM <integer> | Read from the TMC5160 for specific axis motor controller (MPM for pan or MTM for tilt). <integer> is a register address. |
| ### =PSE | Print all current settings to the console. |
| ### =RSE | Refresh current settings by reading then from EEPROM. |
| ### =PMD | Print debug information from TMC5160 controller and internal structures about motors state. |
| ### =SFT <float> | Set fake temperature. To disable this feature you need to set NaN or any number below -100.0 or any number above 200.0. Fake temperature will work only if provided value will be in range from -100.0 to 200.0 |