Command format	3

Device G – General	4

Channel S – System	4

Feature V – Version	4

Feature U – Update	4

Feature R – System Reset	5

Feature S – Cold Start	5

Feature W – Keep Warm	5

Device P – Power	6

Channel – 0 (Power In); 1 … 3 – (Power Out)	6

Feature C – Control	6

Feature V – Voltage	6

Feature I – Current	6

Feature T – Temperature	6

Feature W – Overheat Warning Temperature	6

Feature Z – Overheat Shutdown Temperature	6

Device S – Switch	7

Channel – 1 … 3	7

Feature C – Control	7

Feature D – Duty	7

Device F – Fan	8

Channel – 1	8

Feature C – Control	8

Feature D – Duty	8

Feature V – Voltage	8

Feature I – Current	8

Device X – Expansion Port	9

Channel – 1.0 … 1.3 or 2.0 … 2.3	9

Feature G – Get State	9

Feature S – Set State	9

Feature T – Toggle	9

Feature O – Open-Drain Output	9

Feature P – Push-Pull Output	9

Device E – Environment	10

Channel – R (Remote)	10

Channel – L (Local)	10

Feature T – Get Temperature	10

Feature H – Get Humidity	10

Feature N – Temperature Setpoint	10

Feature O – Temperature Setpoint Overshoot	10

Examples	11

# Command format

The interface is via UART (TTL logic levels) at 9600 bps, 8N1 protocol. The Companion board is controlled with text commands issued by from the host.

Any command line starts with a '=' character and spaces and finishes with '\r' or '\n' or both.

=   [option ?]   [device]   [channel]   [feature]   [[...parameter(s)...]]   [[\checksum]]   [ '\r' '\n' ]

Only one command per command line is allowed.

Spaces leading or trailing the command line, as well as spaces between the various parts in it as ignored.

The ‘?’ option indicates that the command is of query type and should not perform any action but only return the current status of the addressed feature.

The host can optionally provide a checksum to the command line. The checksum is calculated as an 8-bit result from all bytes in the command line, by the formula F(b) = (F' ^ b) + 0xA5, where F' is the current accumulated result, starting with the ‘=’ character (any leading spaces before it must be ignored), and including all characters, plus the ‘\’ character. Reference for software development:

#define addChkByte(result, b) { result = (uint8_t) (((result) ^ (b)) + 0xA5); }

The checksum is provided in the fixed form of a 2-digit hexadecimal number.

Responses are in the form:

>   [command string]   :   [result]   [\checksum]   [ '\r' '\n' ]

Checksum in responses is always provided.

Depending on the command, one or more response strings may be returned.

Commands and queries which are impossible to execute, are ignored, and an empty response string is returned.

# Device G – General

## Channel S – System

### Feature V – Version

(no parameters)

[query] Return version information string

### Feature U – Update

(no parameters)

[set/query] Invoke the software update bootloader. When executed as query, return version information string

When in bootloader mode, a limited different set of commands are available:

=R	Reset

=E	Erase the entire user app area (automatically performed before =U if necessary)

=U	Start uploading user app via XMODEM protocol (128-byte packet)

=L  [[address]]	 List flash memory until Ctrl-C (Space key can be used to pause/resume output)

All bootloader responses follow the same format shown below:

>   [message]   [\checksum]   [ '\r' '\n' ]

A message “>XMODEM” serves as a prompt to the host to initiate XMODEM transmission.

All operations finish with message “>ERRORS:nnn”, where ‘nnn’ is the number of errors during the operation.

Bootloader Error Codes

D     Invalid data format

C     Cancelled

Y     Sync error

F     Failed retry

W   Flash writing error

K     Invalid checksum

A     Invalid linear address

U     Unsupported/unknown

### Feature R – System Reset

(no parameters)

[set] Force a system reset

### Feature S – Cold Start

(flag)

[set/query] Accepted parameters are 1 (ON), or 0 (OFF).

Enable or disable cold start functionality. Cold start functionality is active when all output power channels are switched off. By default, the system starts with it enabled, and can be disabled by the host within the first 30 seconds. Later, if the host switches off all power outputs, the cold start functionality will be off, unless it is explicitly turned on again with this command.

=GSS1 TURN ON

=?GSS CHECK STATUS

=GSS0 TURN OFF

The cold start functionality is enabled by default.

### Feature W – Keep Warm

(flag)

[set/query] Accepted parameters are 1 (ON), or 0 (OFF).

Enable or disable “keep warm” functionality. The keep warm functionality is similar to cold start, however, it disregards the state of the power outputs and only maintains the temperature as per the set parameter for cold start.

The keep warm functionality is enabled by default.

# Device P – Power

## Channel – 0 (Power In); 1 … 3 – (Power Out)

### Feature C – Control

(state)

[set/query] Accepted parameters are 2 (CYCLE/DELAY), 1 (ON), or 0 (OFF).

Enable or disable a specified power output.

When a parameter 2 is provided, the channel is initially forced off, and then forced on after a 5-second delay.

Note: Channel 0 cannot be controlled. It is always on.

### Feature V – Voltage

(no parameters)

[query] Return measured voltage (in Volts) on the specified channel.

### Feature I – Current

(no parameters)

[query] Return measured current (in Amps) on the specified channel.

### Feature T – Temperature

(no parameters)

[query] Return measured temperature (in degrees Celsius) on the specified channel.

### Feature W – Overheat Warning Temperature

(temperature in degrees Celsius)

[set/query] Temperature for overheat warning on the channel.

Can be NAN do disable the function.

### Feature Z – Overheat Shutdown Temperature

(temperature in degrees Celsius)

[set/query] Temperature for overheat shutdown on the channel. If the channel was on, it will resume work once its temperature falls below the overheat warning level.

Can be NAN do disable the function.

# Device S – Switch

## Channel – 1 … 3

### Feature C – Control

(state)

[set/query] Accepted parameters are 1 (ON), or 0 (OFF).

Enable or disable a specified switch.

### Feature D – Duty

(duty in %)

[set/query] Accepted parameters are integer number between 0 and 100.

Set duty cycle (in percent) of an output. The carrier frequency is fixed at 100 Hertz.

# Device F – Fan

## Channel – 1

### Feature C – Control

(state)

[set/query] Accepted parameters are 1 (ON), or 0 (OFF).

Enable or disable a specified switch.

### Feature D – Duty

(duty in %)

[set/query] Accepted parameters are integer number between 0 and 100.

Set duty cycle (in percent) of an output. The carrier frequency is fixed at 100 Hertz.

### Feature V – Voltage

(no parameters)

[query] Return measured voltage (in Volts) on the specified channel.

### Feature I – Current

(no parameters)

[query] Return measured current (in Amps) on the specified channel.

# Device X – Expansion Port

## Channel – 1.0 … 1.3 or 2.0 … 2.3

### Feature G – Get State

(no parameters)

[query] Return the read state of the port as an integer number 0 or 1.

### Feature S – Set State

(state)

[set/query] Accepted parameters are 0 or 1.

Set port in a specified digital state. When executed as query, will return the desired state of the port (vs. the actual state, got by the ‘G’ feature).

### Feature T – Toggle

(no parameters)

[set/query] Toggle the digital state of a port. When executed as query, only returns the current state of the port.

### Feature O – Open-Drain Output

(no parameters)

[set/query] Enable Open-Drain output on the port.

### Feature P – Push-Pull Output

(no parameters)

[set/query] Enable Push-Pull output on the port. This is the default state of all digital outputs.

# Device E – Environment

## Channel – R (Remote)

The ' Remote' channel may include one or more sensor device, connected on a Single-Wire serial channel. Hence, the 'R' character must be followed by an address specifying a device on the bus. The accepted numbers are between 1 and the detected number of devices on the bus.

When a command for the remote channel is received, it is relayed immediately to the devices on the bus, and responses coming from them, relayed back to the host.

## Channel – L (Local)

The ' Local' channel is I2C interface and supports SHT40 sensor.

### Feature T – Get Temperature

(no parameters)

[query] Get temperature in degrees Celsius from a SHT40 sensor on the local bus. If no sensor is detected, will return “NAN” value.

### Feature H – Get Humidity

(no parameters)

[query] Get humidity level in percent from a SHT40 sensor on the local bus. If no sensor is detected, will return “NAN” value.

### Feature N – Temperature Setpoint

(temperature goal in degrees Celsius or .NAN to disable)

[set/query] Set/get temperature goal in degrees Celsius. The board will use the allocated switch channels to control heaters in attempt to reach and maintain this temperature goal.

### Feature O – Temperature Setpoint Overshoot

(positive temperature delta in degrees Celsius or .NAN to disable)

[set/query] Set/get temperature overshoot in degrees Celsius. The temperature goal with added offset from the overshoot defines the point at which the heaters become fully off.

# Examples

Note: Spaces in the provided command strings are only for clarity and can be ignored in actual command strings sent to the controller.

= ? P 2 V	- Check voltage on power output 2
			Command marker / Query / Power / Channel 2 / Voltage

= P 1 C 1	- Enable power output 1
			Command marker / Power / Channel 1 / Control / Enable

= F 1 D 50	- Set duty cycle 50% of fan 1
			Command marker / Fan / Channel 1 / Duty / 50

= F 1 C 1	- Enable fan 1
			Command marker / Fan / Channel 1 / Control / Enable

= X 1.2 S 1	- Set port 1.2 in digital state 1
			Command marker / Expansion Port / Channel 1.2 / Set / 1 (high)

= ? E R1 T	- Check temperature from remote sensor 1 
(assuming 'T' is a valid command of the connected device)